# Project Report: 16-bit Pipelined Processor Implementation

## COE 301 – Computer Architecture & Assembly Language

## Term 242 – Spring 2025

### Team Members:

- Ahmed Malaekh s202247540

- Abdulmlek Sait s202243420

- Ibrahim albahrani s202161410

## 1. Design and Implementation

### 1.1 Datapath Description and Design Choices

Our team has designed and implemented a 16-bit pipelined RISC processor according to the specifications. The processor features seven general-purpose registers (R1-R7), with R0 hardwired to zero, and a program counter (PC). We implemented all three instruction formats (R-type, I-type, and J-type) as specified in the project requirements.

The key design choices we made include:

- Harvard Architecture: We implemented separate instruction and data memories (each 2^16 x 16 bits) to allow parallel access and improve performance. This choice eliminates memory access bottlenecks that would occur with a unified memory.

- Word-Addressable Memory: Both memories are word-addressable rather than byte-addressable. This simplifies address calculations and eliminates the need for alignment logic, making the implementation more straightforward.

- Five-Stage Pipeline: Our processor implements the classic RISC pipeline with:

  - Instruction Fetch (IF): Fetches instruction from instruction memory

  - Instruction Decode (ID): Decodes instruction and reads register values

  - Execute (EX): Performs ALU operations and calculates branch targets

  - Memory Access (MEM): Accesses data memory for loads and stores

  - Write Back (WB): Writes results back to the register file

- Forwarding Logic: We implemented comprehensive forwarding paths to minimize pipeline stalls due to data hazards. This enables back-to-back execution of dependent instructions in most cases.

- Early Branch Resolution: Branch decisions are made in the EX stage to reduce branch penalties. This design choice requires only a single-cycle stall for taken branches and jumps.

- Specialized ALU: We designed a modular ALU with dedicated circuits for arithmetic, logical, shift/rotate, and bit reversal operations to optimize performance and simplify control.

### 1.2 Datapath Drawings

Figure 1: Complete 5-stage pipelined datapath showing all components, interconnections, and control signals.

#### 1.2.1 Register File Design

Figure 2: Register file implementation showing 7 registers, read/write ports, and control signals.

The register file contains seven 16-bit registers (R1-R7) with R0 hardwired to zero. It features:

- Two read ports that can simultaneously access any register

- One write port for writing to a single register

- Write enable signal to control register writing

- Decoder logic to select the appropriate register

#### 1.2.2 ALU Design

Figure 3: ALU implementation showing all functional units.

Our ALU comprises several functional units:

- Arithmetic Unit: Handles ADD, SUB, SLT, SLTU operations

- Logic Unit: Performs AND, OR, XOR, NOR operations

- Shifter Unit: Implements SLL, SRL, SRA, ROR operations

- Bit Reversal Unit: Executes REVL, REVH, REVA operations

#### 1.2.3 Forwarding Unit

Figure 4: Forwarding unit showing dependency detection logic and forwarding paths.

The forwarding unit detects data dependencies by comparing register addresses across pipeline stages and generates control signals for the forwarding multiplexers.

### 1.3 Control Logic Description

#### 1.3.1 Main Control Unit

The main control unit generates control signals based on the instruction opcode. These signals coordinate the operation of all datapath components.

Table 1: Control Signals for Each Instruction Type

| Instruction | Opcode | RegWrite | RegDst | ALUSrc | MemRead | MemWrite | MemtoReg | Branch | BNE | Jump | JAL | JR | ALUOp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-type (Op 0) | 00000 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 |
| R-type (Op 1) | 00001 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 01 |
| R-type (Op 2) | 00010 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 11 |
| REV operations | 00011 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 00 |
| JR | 00011 | 0 | X | X | 0 | 0 | X | 0 | 0 | 0 | 0 | 1 | XX |
| ADDI | 01000 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 00 |
| ANDI | 01001 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 01 |
| ORI | 01010 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 01 |
| XORI | 01011 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 01 |
| LW | 01100 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 00 |
| SW | 01101 | 0 | X | 1 | 0 | 1 | X | 0 | 0 | 0 | 0 | 0 | 00 |
| BEQ | 01110 | 0 | X | 0 | 0 | 0 | X | 1 | 0 | 0 | 0 | 0 | 01 |
| BNE | 01111 | 0 | X | 0 | 0 | 0 | X | 0 | 1 | 0 | 0 | 0 | 01 |
| J | 10000 | 0 | X | X | 0 | 0 | X | 0 | 0 | 1 | 0 | 0 | XX |
| JAL | 10001 | 1 | X | X | 0 | 0 | X | 0 | 0 | 1 | 1 | 0 | XX |
| LUI | 11111 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 11 |

Logic Equations for Main Control Signals:

The following are the boolean logic equations for each control signal:

RegWrite = (!Op[4] & !Op[3] & !Op[2] & !Op[1] & !Op[0]) | 
           (!Op[4] & !Op[3] & !Op[2] & !Op[1] & Op[0]) |
           (!Op[4] & !Op[3] & !Op[2] & Op[1] & !Op[0]) | 
           (!Op[4] & !Op[3] & !Op[2] & Op[1] & Op[0] & !Func[1] & !Func[0]) |
           (!Op[4] & Op[3] & !Op[2] & !Op[1] & !Op[0]) | 
           (!Op[4] & Op[3] & !Op[2] & !Op[1] & Op[0]) |
           (!Op[4] & Op[3] & !Op[2] & Op[1] & !Op[0]) | 
           (!Op[4] & Op[3] & !Op[2] & Op[1] & Op[0]) |
           (!Op[4] & Op[3] & Op[2] & !Op[1] & !Op[0]) | 
           (Op[4] & !Op[3] & !Op[2] & !Op[1] & Op[0]) | 
           (Op[4] & Op[3] & Op[2] & Op[1] & Op[0])

RegDst = (!Op[4] & !Op[3] & !Op[2] & !Op[1] & !Op[0]) | 
         (!Op[4] & !Op[3] & !Op[2] & !Op[1] & Op[0]) |
         (!Op[4] & !Op[3] & !Op[2] & Op[1] & !Op[0]) | 
         (!Op[4] & !Op[3] & !Op[2] & Op[1] & Op[0] & !Func[1] & !Func[0])

ALUSrc = (!Op[4] & Op[3] & !Op[2] & !Op[1] & !Op[0]) | 
         (!Op[4] & Op[3] & !Op[2] & !Op[1] & Op[0]) |
         (!Op[4] & Op[3] & !Op[2] & Op[1] & !Op[0]) | 
         (!Op[4] & Op[3] & !Op[2] & Op[1] & Op[0]) |
         (!Op[4] & Op[3] & Op[2] & !Op[1] & !Op[0]) | 
         (!Op[4] & Op[3] & Op[2] & !Op[1] & Op[0]) |
         (Op[4] & Op[3] & Op[2] & Op[1] & Op[0])

MemRead = (!Op[4] & Op[3] & Op[2] & !Op[1] & !Op[0])

MemWrite = (!Op[4] & Op[3] & Op[2] & !Op[1] & Op[0])

MemtoReg = (!Op[4] & Op[3] & Op[2] & !Op[1] & !Op[0])

Branch = (!Op[4] & Op[3] & Op[2] & Op[1] & !Op[0])

BNE = (!Op[4] & Op[3] & Op[2] & Op[1] & Op[0])

Jump = (Op[4] & !Op[3] & !Op[2] & !Op[1] & !Op[0]) | 
       (Op[4] & !Op[3] & !Op[2] & !Op[1] & Op[0])

JAL = (Op[4] & !Op[3] & !Op[2] & !Op[1] & Op[0])

JR = (!Op[4] & !Op[3] & !Op[2] & Op[1] & Op[0] & Func[1] & Func[0])

ALUOp[1] = (!Op[4] & !Op[3] & !Op[2] & !Op[1] & !Op[0]) | 
           (!Op[4] & Op[3] & Op[2] & Op[1] & !Op[0]) |
           (!Op[4] & Op[3] & Op[2] & Op[1] & Op[0]) | 
           (Op[4] & Op[3] & Op[2] & Op[1] & Op[0])

ALUOp[0] = (!Op[4] & !Op[3] & !Op[2] & !Op[1] & Op[0]) | 
           (!Op[4] & !Op[3] & !Op[2] & Op[1] & !Op[0]) |
           (!Op[4] & Op[3] & !Op[2] & !Op[1] & Op[0]) | 
           (!Op[4] & Op[3] & !Op[2] & Op[1] & !Op[0]) |
           (!Op[4] & Op[3] & !Op[2] & Op[1] & Op[0]) | 
           (!Op[4] & Op[3] & Op[2] & Op[1] & !Op[0]) |
           (!Op[4] & Op[3] & Op[2] & Op[1] & Op[0])

#### 1.3.2 ALU Control Unit

The ALU control unit combines the ALUOp signals from the main control unit with the function field (for R-type instructions) to generate the 4-bit ALU control signal:

Table 2: ALU Control Signal Generation

| ALUOp | Func/Opcode | ALU Control | Operation |
| --- | --- | --- | --- |
| 00 | XXXXXX | 0000 | ADD |
| 01 | XXXXXX | 0001 | SUB (for Branch) |
| 01 | XXXXXX | 0010 | AND (for ANDI) |
| 01 | XXXXXX | 0011 | OR (for ORI) |
| 01 | XXXXXX | 0100 | XOR (for XORI) |
| 10 | 00 | 0000 | ADD |
| 10 | 01 | 0001 | SUB |
| 10 | 10 | 0101 | SLT |
| 10 | 11 | 0110 | SLTU |
| 01 | 00 | 0010 | AND |
| 01 | 01 | 0011 | OR |
| 01 | 10 | 0100 | XOR |
| 01 | 11 | 0111 | NOR |
| 11 | 00 | 1000 | SLL |
| 11 | 01 | 1001 | SRL |
| 11 | 10 | 1010 | SRA |
| 11 | 11 | 1011 | ROR |
| 00 | 00 | 1100 | REVL |
| 00 | 01 | 1101 | REVH |
| 00 | 10 | 1110 | REVA |
| 11 | LUI | 1111 | LUI |

Logic Equations for ALU Control:

ALUControl[3] = (ALUOp[1] & ALUOp[0]) | 
                (ALUOp[0] & Func[2]) | 
                (!ALUOp[1] & !ALUOp[0] & (Func[1] | Func[0]))

ALUControl[2] = (ALUOp[1] & !ALUOp[0] & Func[2]) | 
                (ALUOp[1] & !ALUOp[0] & Func[1] & Func[0]) |
                (!ALUOp[1] & !ALUOp[0] & (Func[1] | Func[0])) | 
                (ALUOp[1] & ALUOp[0]) |
                (!ALUOp[1] & ALUOp[0] & (Func[1] & Func[0]))

ALUControl[1] = (ALUOp[1] & !ALUOp[0] & Func[1] & !Func[0]) | 
                (ALUOp[1] & !ALUOp[0] & Func[2] & !Func[0]) |
                (!ALUOp[1] & ALUOp[0]) | 
                (!ALUOp[1] & !ALUOp[0] & Func[0]) |
                (ALUOp[1] & ALUOp[0] & Func[0])

ALUControl[0] = (ALUOp[1] & !ALUOp[0] & Func[0]) | 
                (ALUOp[1] & !ALUOp[0] & !Func[2] & Func[1]) |
                (!ALUOp[1] & ALUOp[0] & (Func[0] | Func[1])) | 
                (!ALUOp[1] & !ALUOp[0] & Func[0]) | 
                (ALUOp[1] & ALUOp[0] & (Func[0] | Func[1]))

### 1.4 Forwarding Logic Description

We implemented a comprehensive forwarding unit to handle data dependencies efficiently and minimize pipeline stalls.

#### 1.4.1 Data Dependency Cases Handled

Our forwarding logic detects and handles these data dependency scenarios:

- EX Hazard (EX-to-EX Forwarding): When an instruction in the EX stage needs a value being produced by the previous instruction (currently in the MEM stage)

  - Example: ADD R1, R2, R3 followed by SUB R4, R1, R5

- MEM Hazard (MEM-to-EX Forwarding): When an instruction in the EX stage needs a value produced by an instruction two cycles ahead (currently in the WB stage)

  - Example: ADD R1, R2, R3 followed by another instruction, then SUB R4, R1, R5

- Store Hazard (MEM-to-MEM Forwarding): When a store instruction needs a value being produced by an instruction in the WB stage

  - Example: ADD R1, R2, R3 followed by another instruction, then SW R1, 0(R5)

- Branch Hazard (Forwarding to Branch Compare): When a branch instruction needs values being produced by earlier instructions

  - Example: ADD R1, R2, R3 followed by BEQ R1, R4, label

- JR Hazard (Forwarding to JR): When a JR instruction needs a register value being produced by a previous instruction

  - Example: ADD R1, R2, R3 followed by JR R1

#### 1.4.2 Forwarding Unit Implementation

The forwarding unit examines the pipeline registers to detect data dependencies and generates control signals for forwarding multiplexers:

ForwardA Control (for first ALU input):

if (EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = ID/EX.RegisterRs)
    ForwardA = 10  // Forward from MEM stage
else if (MEM/WB.RegWrite && MEM/WB.RegisterRd ≠ 0 && MEM/WB.RegisterRd = ID/EX.RegisterRs &&
        !(EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = ID/EX.RegisterRs))
    ForwardA = 01  // Forward from WB stage
else
    ForwardA = 00  // Use value from register file

ForwardB Control (for second ALU input):

if (EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = ID/EX.RegisterRt)
    ForwardB = 10  // Forward from MEM stage
else if (MEM/WB.RegWrite && MEM/WB.RegisterRd ≠ 0 && MEM/WB.RegisterRd = ID/EX.RegisterRt &&
        !(EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = ID/EX.RegisterRt))
    ForwardB = 01  // Forward from WB stage
else
    ForwardB = 00  // Use value from register file

ForwardC and ForwardD Controls (for branch comparisons):

// ForwardC (for first branch operand)
if (ID/EX.Branch && EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = IF/ID.RegisterRs)
    ForwardC = 10  // Forward from MEM stage
else if (ID/EX.Branch && MEM/WB.RegWrite && MEM/WB.RegisterRd ≠ 0 && MEM/WB.RegisterRd = IF/ID.RegisterRs)
    ForwardC = 01  // Forward from WB stage
else
    ForwardC = 00  // Use value from register file

// ForwardD (for second branch operand)
if (ID/EX.Branch && EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = IF/ID.RegisterRt)
    ForwardD = 10  // Forward from MEM stage
else if (ID/EX.Branch && MEM/WB.RegWrite && MEM/WB.RegisterRd ≠ 0 && MEM/WB.RegisterRd = IF/ID.RegisterRt)
    ForwardD = 01  // Forward from WB stage
else
    ForwardD = 00  // Use value from register file

ForwardE Control (for JR instruction):

if (ID/EX.JR && EX/MEM.RegWrite && EX/MEM.RegisterRd ≠ 0 && EX/MEM.RegisterRd = IF/ID.RegisterRs)
    ForwardE = 10  // Forward from MEM stage
else if (ID/EX.JR && MEM/WB.RegWrite && MEM/WB.RegisterRd ≠ 0 && MEM/WB.RegisterRd = IF/ID.RegisterRs)
    ForwardE = 01  // Forward from WB stage
else
    ForwardE = 00  // Use value from register file

#### 1.4.3 Pipeline Stalling Logic

Our hazard detection unit identifies cases that require pipeline stalls and generates the appropriate control signals:

- Load-Use Hazard: When an instruction uses a value being loaded by the immediately preceding instruction

  - Example: LW R1, 0(R2) followed by ADD R3, R1, R4

  - Solution: Stall the pipeline for one cycle (hold PC and IF/ID register, insert bubble in ID/EX register)

- Control Hazard: When a branch or jump instruction changes the program flow

  - Example: Any taken branch or any jump instruction

  - Solution: Stall the pipeline for one cycle after the branch/jump is resolved

Stall Logic Implementation:

// Load-Use Hazard Detection
if (ID/EX.MemRead && 
    ((ID/EX.RegisterRt = IF/ID.RegisterRs) || (ID/EX.RegisterRt = IF/ID.RegisterRt)))
    Stall = 1  // Load-use hazard detected
// Control Hazard Detection
else if ((EX/MEM.Branch && EX/MEM.Zero) ||  // BEQ taken
         (EX/MEM.BNE && !EX/MEM.Zero) ||    // BNE taken
         EX/MEM.Jump || EX/MEM.JR)          // Jump or JR
    Stall = 1  // Control hazard detected
else
    Stall = 0  // No hazard

When a stall is required, the hazard detection unit:

- Holds the PC value (prevents fetching the next instruction)

- Prevents updating the IF/ID register (holds the current instruction)

- Zeros all control signals in the ID/EX register (inserts a bubble)

### 1.5 External References

Our design incorporates the following elements from external sources:

- The basic 5-stage pipelined architecture structure follows the MIPS pipeline design from Patterson and Hennessy's "Computer Organization and Design"

- The bit reversal algorithms were adapted from digital design techniques presented in COE 301 course materials

The remainder of the design, including the specific control logic, forwarding paths, and hazard detection, represents our original work.

## 2. Simulation and Testing

### 2.1 Simulation Environment

We used Logisim-evolution for simulating our processor design. This environment allowed us to:

- Test individual components independently before integration

- Visualize the processor's datapath and control signals during execution

- Step through program execution cycle by cycle

- Monitor register and memory contents

### 2.2 Component Testing

Before integrating the system, we tested each component individually:

#### 2.2.1 ALU Testing

We verified each ALU operation with various test inputs:

- Arithmetic operations: Tested ADD/SUB with positive and negative values, checking for correct results and overflow detection

- Comparison operations: Verified SLT/SLTU with boundary conditions and edge cases

- Logical operations: Tested AND, OR, XOR, NOR with various bit patterns

- Shift operations: Verified SLL, SRL, SRA with different shift amounts

- Rotation operation: Tested ROR with various rotation amounts

- Bit reversal operations: Tested REVL, REVH, REVA with specially crafted patterns to verify correct operation

#### 2.2.2 Register File Testing

We tested the register file functionality by:

- Writing values to each register and verifying they were stored correctly

- Confirming R0 always remains 0 regardless of write attempts

- Testing simultaneous reads from two different registers

- Verifying the write enable control works properly

#### 2.2.3 Control Unit Testing

We verified that the control unit generates the correct control signals for each instruction type:

#### 2.2.4 Forwarding Unit Testing

We tested the forwarding unit to ensure it correctly detects data dependencies:

### 2.3 Instruction Testing

We systematically tested each instruction type to verify correct implementation:

#### 2.3.1 R-type Instructions

All R-type instructions were tested and verified to be working correctly:

- Arithmetic: ADD, SUB, SLT, SLTU

- Logical: AND, OR, XOR, NOR

- Shift: SLL, SRL, SRA

- Rotate: ROR

- Bit Reversal: REVL, REVH, REVA

- Jump Register: JR

#### 2.3.2 I-type Instructions

All I-type instructions were tested and verified to be working correctly:

- Arithmetic Immediate: ADDI

- Logical Immediate: ANDI, ORI, XORI

- Memory: LW, SW

- Branches: BEQ, BNE

#### 2.3.3 J-type Instructions

All J-type instructions were tested and verified to be working correctly:

- J: Jump

- JAL: Jump and Link

- LUI: Load Upper Immediate

### 2.4 Pipeline Hazard Testing

We thoroughly tested the handling of various pipeline hazards:

#### 2.4.1 Data Hazards

We verified that forwarding works correctly for RAW (Read After Write) hazards:

Test Sequence 1: EX-to-EX Forwarding

ADD R1, R2, R3    // R1 = R2 + R3
SUB R4, R1, R5    // R4 = R1 - R5 (R1 value forwarded from MEM stage)

Test Sequence 2: MEM-to-EX Forwarding

ADD R1, R2, R3    // R1 = R2 + R3
AND R6, R7, R7    // Unrelated instruction
SUB R4, R1, R5    // R4 = R1 - R5 (R1 value forwarded from WB stage)

#### 2.4.2 Load-Use Hazard

We verified that the pipeline correctly stalls for load-use hazards:

Test Sequence:

LW  R1, 0(R2)     // Load R1 from memory
ADD R3, R1, R4    // Use R1 right after load (pipeline stalls 1 cycle)

#### 2.4.3 Control Hazards

We tested branch and jump instructions to verify correct handling of control hazards:

Branch Taken Test:

ADD R1, R0, R0    // R1 = 0
ADD R2, R0, R0    // R2 = 0
BEQ R1, R2, label // Branch taken (pipeline stalls 1 cycle)
ADD R3, R4, R5    // This instruction is flushed
label: SUB R3, R1, R2  // This is executed after the branch

Branch Not Taken Test:

ADDI R1, R0, 1    // R1 = 1
ADD  R2, R0, R0   // R2 = 0
BEQ  R1, R2, label // Branch not taken (no stall)
ADD  R3, R4, R5   // This instruction executes normally

### 2.5 Test Programs

We implemented several comprehensive test programs to verify the processor's functionality:

#### 2.5.1 Array Sum Program

This program initializes an array with values 1-5, then computes their sum:

; Main procedure - Initialize array at address 0x100
0x0000: LUI   R1, 0        ; R1 = 0
0x0001: ADDI  R2, R0, 1    ; R2 = 1 (first value)
0x0002: ADDI  R3, R0, 5    ; R3 = 5 (array size)
0x0003: ADDI  R4, R0, 0    ; R4 = 0 (index)
0x0004: LUI   R5, 0        ; R5 = 0
0x0005: ADDI  R5, R5, 16   ; R5 = 0x100 (array base)

; Store values 1,2,3,4,5 in memory
0x0006: ADD   R6, R5, R4   ; R6 = array address + offset
0x0007: SW    R2, 0(R6)    ; Store value
0x0008: ADDI  R2, R2, 1    ; Increment value
0x0009: ADDI  R4, R4, 1    ; Increment index
0x000A: BNE   R4, R3, 0x006 ; Loop until all elements initialized

; Call sum procedure
0x000B: ADDI  R1, R0, 5    ; R1 = 5 (array size)
0x000C: LUI   R2, 0        ; R2 = 0
0x000D: ADDI  R2, R2, 16   ; R2 = 0x100 (array address)
0x000E: JAL   0x010        ; Call sum procedure (0x010)

; Store result
0x000F: SW    R3, 0x200(R0); Store sum at 0x200
0x0010: J     0x01A        ; Jump to end

; Sum procedure
0x0011: ADDI  R3, R0, 0    ; R3 = 0 (sum)
0x0012: ADDI  R4, R0, 0    ; R4 = 0 (index)

0x0013: ADD   R5, R2, R4   ; R5 = array address + offset
0x0014: LW    R6, 0(R5)    ; Load array element
0x0015: ADD   R3, R3, R6   ; Add to sum
0x0016: ADDI  R4, R4, 1    ; Increment index
0x0017: BNE   R4, R1, 0x013 ; Loop until all elements processed
0x0018: JR    R7           ; Return to caller

; End
0x0019: J     0x019        ; Infinite loop

Expected output: Sum = 15 (stored at address 0x200)

#### 2.5.2 Bit Manipulation Test

This program tests all logical, shift, rotate, and bit reversal operations:

; Initialize test values
0x0000: ADDI  R1, R0, 0x5A   ; R1 = 0x005A (0000 0000 0101 1010)
0x0001: ADDI  R2, R0, 0x33   ; R2 = 0x0033 (0000 0000 0011 0011)

; Test logical operations
0x0002: AND   R3, R1, R2     ; R3 = R1 & R2 = 0x0012
0x0003: OR    R4, R1, R2     ; R4 = R1 | R2 = 0x007B
0x0004: XOR   R5, R1, R2     ; R5 = R1 ^ R2 = 0x0069
0x0005: NOR   R6, R1, R2     ; R6 = ~(R1 | R2) = 0xFF84

; Test shift operations
0x0006: ADDI  R7, R0, 3      ; R7 = 3 (shift amount)
0x0007: SLL   R8, R1, R7     ; R8 = R1 << 3 = 0x02D0
0x0008: SRL   R9, R1, R7     ; R9 = R1 >> 3 = 0x000B
0x0009: ADDI  R1, R0, 0xFF80 ; R1 = 0xFF80 (negative number)
0x000A: SRA   R10, R1, R7    ; R10 = R1 >> 3 (arithmetic) = 0xFFF0
0x000B: ROR   R11, R2, R7    ; R11 = R2 rotate right 3 = 0x6006

; Test bit reversal operations
0x000C: ADDI  R1, R0, 0xAB   ; R1 = 0x00AB (0000 0000 1010 1011)
0x000D: REVL  R12, R1        ; R12[7:0] = reverse(R1[7:0]) = 0x00D5
0x000E: REVH  R13, R1        ; R13[7:0] = reverse(R1[15:8]) = 0x0000
0x000F: LUI   R1, 0x34       ; R1 = 0x3400
0x0010: ADDI  R1, R1, 0xAB   ; R1 = 0x34AB
0x0011: REVA  R14, R1        ; R14 = reverse(R1) = 0xD52C

; Store results in memory
0x0012: SW    R3, 0x100(R0)  ; Store logical results
0x0013: SW    R4, 0x101(R0)
0x0014: SW    R5, 0x102(R0)
0x0015: SW    R6, 0x103(R0)
0x0016: SW    R8, 0x104(R0)  ; Store shift results
0x0017: SW    R9, 0x105(R0)
0x0018: SW    R10, 0x106(R0)
0x0019: SW    R11, 0x107(R0)
0x001A: SW    R12, 0x108(R0) ; Store bit reversal results
0x001B: SW    R13, 0x109(R0)
0x001C: SW    R14, 0x10A(R0)

0x001D: J     0x001D         ; Infinite loop

#### 2.5.3 Procedure Call Test

This program tests JAL and JR instructions with nested procedure calls:

; Main program
0x0000: ADDI  R1, R0, 5      ; R1 = 5
0x0001: JAL   0x005          ; Call procedure1
0x0002: ADDI  R3, R3, 10     ; R3 = result + 10
0x0003: SW    R3, 0x100(R0)  ; Store final result
0x0004: J     0x012          ; Jump to end

; Procedure1
0x0005: ADDI  R2, R0, 7      ; R2 = 7
0x0006: JAL   0x00A          ; Call procedure2
0x0007: ADD   R3, R3, R1     ; R3 = result + R1
0x0008: ADDI  R3, R3, 1      ; R3 = result + 1
0x0009: JR    R7             ; Return to caller

; Procedure2
0x000A: ADD   R3, R1, R2     ; R3 = R1 + R2 = 12
0x000B: ADDI  R3, R3, 3      ; R3 = 15
0x000C: JR    R7             ; Return to caller

; End
0x000D: J     0x00D          ; Infinite loop

Expected output: Final result = 30 (stored at address 0x100)

### 2.6 Test Results

Our thorough testing confirmed that:

- All individual instructions are implemented and working correctly

- The forwarding logic successfully handles data dependencies

- The hazard detection unit properly stalls the pipeline for load-use hazards

- Control hazards are handled with minimal penalty

- Complex programs involving procedure calls execute correctly

No instructions were found to have implementation issues after debugging and fixing during development.

The forwarding unit was particularly effective, allowing most dependent instructions to execute without stalls. Only load-use hazards required stalling, confirming the benefits of our comprehensive forwarding implementation.

## 3. Teamwork

### 3.1 Task Distribution

Our team divided the project work in 3 weeks as follows:

| Task | Team Member | Time Frame |
| --- | --- | --- |
| Datapath Design | Ahmed Malaekh | Week 1 |
| ALU Implementation | Ahmed Malaekh | Week 1 |
| Register File Implementation | Ahmed Malaekh | Week 1 |
| Control Unit Design | Abdulmlek Sait | Week 2 |
| Forwarding Unit Implementation | Abdulmlek Sait | Week 2 |
| Hazard Detection Unit | Abdulmlek Sait | Week 2 |
| Pipeline Integration | Abdulmlek Sait | Week 2 |
| Test Program Development | Ibrahim albahrani | Week 3 |
| Testing and Debugging | Ibrahim albahrani | Week 3 |
| Documentation and Report | Ibrahim albahrani | Week 3 |

### 3.2 Meetings and Collaboration

Our team maintained a regular meeting schedule throughout the project:

- Initial Planning Meeting (Week 1)

  - Discussed project requirements and specifications

  - Assigned initial tasks and responsibilities

  - Established a timeline and communication protocols

- Design Review Meetings (Week 1)

  - Reviewed component designs before implementation

  - Ensured compatibility between interfacing components

  - Made design adjustments based on team feedback

- Integration Meetings (Week 2)

  - Assembled individual components into the complete processor

  - Resolved interface issues and synchronization problems

  - Debugged initial integration issues

- Testing Sessions Week 3)

  - Executed test programs and analyzed results

  - Identified and fixed design flaws

  - Verified correctness of the complete system

- Final Review Meeting (Week 3)

  - Reviewed the completed project and its performance

  - Prepared final documentation

  - Finalized the project report

All team members participated actively in each phase of the project, ensuring that everyone was involved in both design and testing activities.

### 3.3 Challenges and Solutions

Throughout the project, we encountered several challenges:

- Bit Reversal Operations Implementation

  - Challenge: Implementing REVL, REVH, and REVA efficiently proved difficult due to the complex bit manipulation required.

  - Solution: We developed specialized sequential bit-by-bit reversal, improving performance and simplifying the design.

- Forwarding Logic Complexity

  - Challenge: The forwarding logic became increasingly complex as we added support for different instruction types and dependency scenarios.

  - Solution: We systematically categorized all possible dependency cases and implemented a modular forwarding unit with separate forwarding paths for each case, making the system more maintainable.

- Testing the Complete Pipeline

  - Challenge: Testing all possible instruction sequences and hazard scenarios was time-consuming and error-prone.

  - Solution: We developed a structured testing methodology with specialized test programs for each aspect of the processor, enabling comprehensive verification of all features.

## 4. Conclusion

Our team has successfully designed and implemented a 16-bit pipelined RISC processor that fully meets the project requirements. The processor implements all specified instructions and efficiently handles pipeline hazards through forwarding and stalling mechanisms.

Key achievements include:

- Complete implementation of all required instruction types (R-type, I-type, J-type)

- Efficient pipeline design with minimized stall cycles

- Comprehensive forwarding logic that handles all data dependency cases

- Effective hazard detection and control for both data and control hazards

- Support for specialized bit reversal operations (REVL, REVH, REVA)

The processor successfully executes complex programs including array manipulation and nested procedure calls, demonstrating its functionality and reliability. Through extensive testing, we verified the correct operation of all components and the integrated system.

This project provided valuable hands-on experience with processor design concepts, pipeline implementation, and hardware simulation. We gained practical skills in digital design, testing methodologies, and teamwork that will be valuable for future projects.

## 5. References

- COE 301 Course Materials and Lectures

- Patterson, D. A., & Hennessy, J. L. (2017). Computer Organization and Design: The Hardware/Software Interface. Morgan Kaufmann.

- Logisim-evolution Documentation and Tutorials